George Zyvoloski.  October 28, 2025

This update is part of a larger document describing the recent developments for AWH. All looks good. I will be implementing the gas phase non-Darcy flow model next.

Test Suite for AWH with Non-darcy Flow (Nov 11, 2025)

- 1D tests

  - Fluids

    - Liquid only (dissolved GAS)

    - Multi-phase

    - GAS only (air and CO2 not yetCO2 <<<)

  - BCs

    - Constant Pressure

    - Constant Flow

  - GDPM and GDKM (not yet <<)

- 2D-3D (not yet <<)

  - Structured

  - Unstructured

- Super critical P and T (not yet <<)

Testing AWH with Non-darcy Flow (Oct 28, 2025)

The coding for the liquid phase with AWH is complete. The simulations described below utilize the 1D problem outlined above to test the supercritical tables and enhanced phase-change algorithms. Figure A4 shows the solution domain and its physical parameters. Note that the thermal diffusivity was set to a low value (3.0e-6 W/(m°C)) so that the advection terms could be isolated. (The code ran fine when the thermal diffusivity was set to 3.0W/(m°C)).) The left boundary conditions consisted of a constant total pressure of 2 Mpa, a noncondensible gas flow injection rate of 1.e-8 kg/sec and a constant temperature of 120 C. The right boundary conditions consisted of a constant total pressure of 1 Mpa and a constant temperature of 80 C. Initial conditions were 1. Mpa total pressure, 0.05 Mpa partial gas pressure, and 80 °C temperature. The total simulation time was 15 days, enough that the Dary flow reached steady state. We note here that the parameters were chosen to prevent phase changes.

Permeability = 1.0e-12 m**2 (unless otherwise noted)

Porosity = 0.1

Thermal diffusivity = 3.0E-6 W/(M*C)

Linear relative permeability model

Linear capillary pressure model (max cap pressure = 0.2 Mpa)

Right BCs

Left BCs

(0,0)                                 50 gridblocks (1 m width)                                 (50,0)

Figure A4.  1D domain used for testing the high-temperature tables.

| Run ID | Left BCs (constant Ptot, T, Gflow) | RightBCs (constant Ptot,T) | ND coef. | Ptot (initial) | T (initial) | Phase (initial) |
| --- | --- | --- | --- | --- | --- | --- |
| ND00 | (2,120,1.e-8) | (1,80) | 0 | 1 | 80 | liquid |
| ND11 | (2,120,1.e-8) | (1,80) | 1.e11 | 1 | 80 | liquid |
| ND12 | (2,120,1.e-8) | (1,80) | 1.e12 | 1 | 80 | liquid |

Table A3. Boundary and initial condition for AWH runs for Darcy flow and non-Darcy flow. Ptot is the Total Fluid pressure (Mpa), T is the temperature (C), Gflow is the noncondensable gas flow(kg/sec). ND00 is the Darcy Flow, ND11 and ND12 are non-Darcy flows (coefficients shown in the table).

Comparisons of the runs are given in Figures A5 and A6. Figure A5 shows the spatial variation of temperature for the runs described in Table A3. Note the nearly constant temperature of ND00, the Darcy flow. Runs ND11 and ND12 produced very different temperatures. This test was designed to highlight the velocity changes that occur with non-Darcy pressure velocity relationships. By applying fixed pressure boundary conditions, the flow rates, 7.98E-02 kg/s for Darcy flow, 1.27214E-02 kg/s for ND11, and 4.29779E-03 kg/s for ND12, are responsible for the differences in energy transport.  Hence, the large temperature differences. The

Figure A5. Temperature distribution at 15 days for Darcy and non-Darcy flow models.

The noncondensable gas pressure at 15 days for the flow models is shown in Figure A6. It should be noted that the gas pressure is a surrogate for dissolved gas in the liquid phase and can be converted to concentration with Henry’s law. Note also the log scale and the differences between flow models.  Given that the injection of gas on the left boundary is the same for all flow models, the flow rate differences allow the gas to accumulate near the left boundary in the ND11 and ND12 models and advect at a slower rate than the ND00 model and proportional to the flow rate.

Figure A6. noncondensable gas pressure distribution at 15 days for Darcy and non-Darcy flow models.

To complete the testing, the numerical performance of the models was compared. Table A4

| Run ID | Newton-Raphson iterations | Solver iterations | Time steps | MBE (H2O) | EBE | MBE (gas) |
| --- | --- | --- | --- | --- | --- | --- |
| ND00 | 21 | 42 | 11 | 1E-07 | 1E-04 | 1e-04 |
| ND11 | 39 | 78 | 11 | 1E-07 | 1E-06 | 1e-05 |
| ND12 | 47 | 94 | 11 | 1E-06 | 1E-07 | 1e-05 |

Table A4. Computational efficiency of non-Darcy flow

shows that the models all have excellent mass and energy balances. There is a gradual increase in the number of iterations. This is expected as the nonlinear part of the non-Darcy velocity model becomes more important as the ND coefficient increases.

Concluding remarks.

- The results are encouraging and allow this author to have confidence that the non-Darcy velocity model has been implemented correctly including the many function derivatives associated with the Newton-Raphson method.

- Though not reported here, many different parameter combinations were explored and judged to be realistic. The author does note that some of the gas pressures tested, representing dissolved gas, were certainly larger than any reasonable solubility limit. (However, the runs all finished well.)

Testing AWH with Non-darcy Flow (Nov 11, 2025)

The coding for Non-Darcy flow AWH is complete for liquid, gas, and multi-phase.

The tests for this set of fluid states will use the same 1D geometry rock parameters described in the liquid-only test with the exception of the permeability, which is an order of magnitude smaller.   The boun macro was used to change water and gas flow rates to produce all phase combinations. A large non-Darcy coefficient (1.0E12) was used to ensure the numerical stability of non-Darcy formulation.

Permeability = 1.0e-13 m**2 (unless otherwise noted)

Porosity = 0.1

Thermal diffusivity = 3.0E-6 W/(M*C)

Linear relative permeability model

Linear capillary pressure model (max cap pressure = 0.2 Mpa)

Right BCs

Left BCs

(0,0)                                 50 gridblocks (1 m width)                                 (50,0)

Figure A6.  1D domain used for testing the high-temperature tables.

Table A5 summarizes the boundary and initial conditions. The initial conditions and the left boundary conditions start the simulation in a two-phase state and show an increasing saturation with time. At 25 days, the water flow rate is increased, and the gas flow rate is decreased, producing a liquid-only state.  At 50 days, the temperature BC is changed from 120 C to 250 C, allowing the system to quickly change to a gas-only condition.

| Time (days) | Left BCs (Wflow(kg/s), T(C), Gflow(kg/s)) | Right BCs (constant Ptot(Mpa), T(C)) | Pg (initial) | Ptot (initial) | T (initial) | Saturation (initial) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0 | (1.e-3,120,1.e-6) | (0.1,90) | 0.01 | 0.2 | 119.5 | 0.1 |
| 25 | (1.e-2,120,1.e-9) | (0.1,90) | NA | NA | NA | NA |
| 50 | (1.e-4,250,1.e-8) | (0.1,90) | NA | NA | NA | NA |

Table A5. Boundary and initial conditions for AWH runs for Darcy flow and non-Darcy flow. Ptot is the Total Fluid pressure (Mpa), Pg is the gas pressure, T is the temperature (°C), Wflow(kg/s) is the water flow Gflow is the noncondensable gas flow. The initial temperature is the boiling temperature at the vapor pressure (Ptot-Pg).

The results for the Darcy and non-Darcy flow are given in Figures A7 to A10.

Figure A7. Pressure vs time of Darcy (ND00) and non-Darcy flow (ND12)

The disparity in pressure is huge in the liquid-only region from 25 days to 50 days. The other time segments are dominated by gas flow, characterized by its lower fluid density and viscosity.

Figure A8 Saturation vs time for Darcy (ND00) and non-Darcy flow (ND12)

Saturation differences between Darcy and non-Darcy flows (Figure A8) occur only in the first 25 days. The differences are due to the lower liquid flow rate on the right boundary of the non-Darcy simulation (1.6E-4 kg/s) vs the Darcy simulation (2.1E-4 kg/s). This allows more liquid water to be retained.

Figure A9 Saturation vs time for Darcy (ND00) and non-Darcy flow (ND12)

The noncondensable gas pressure behavior, shown in Figure A9, is similar to that of the total pressure.  As with the total pressure, the slower non-Darcy flow rate allows the noncondensable gas to discharge more slowly. Figures A10, A11, and A12 provide  the spatial variability of the temperature, pressure, and saturation at different times corresponding to BC changes.  Figure A10 shows  temperature differences that are relatively mild and are dominated by BC temperature changes on the left boundary.

Figure A10. Temperature Vs distance for Darcy (ND00) and non-Darcy flow (ND12)

The pressure results, Figure A11, show that the non-Darcy pressure drop at 50 days with liquid-only conditions is about six times larger than the Darcy pressure drop. See also Figure A7.

Figure A11. Pressure Vs distance for Darcy (ND00) and non-Darcy flow (ND12)

The spatial saturation results, Figure A12, show complexity. For example, even though the time series for the left side boundary node, Figure A8, shows gas only and zero saturation, the spatial distribution is far different.

Figure A12. Saturation vs distance for Darcy (ND00) and non-Darcy flow (ND12)

Lastly, Table A6 shows the progression from Darcy flow (ND00) to non-Darcy flow with a very large ND coefficient (ND12).  It should be noted here that the ND coefficients vary by 3 orders of magnitude from ND09 to ND12.

| Run ID | Newton-Raphson iterations | Solver iterations | Time steps | MBE (H2O) | EBE | MBE (gas) |
| --- | --- | --- | --- | --- | --- | --- |
| ND00 | 349 | 698 | 93 | 1E-06 | 1E-05 | 1e-05 |
| ND09 | 342 | 684 | 93 | 1E-06 | 1E-05 | 1e-05 |
| ND11 | 427 | 854 | 110 | 1E-07 | 1E-07 | 1e-04 |
| ND12 | 663 | 1326 | 140 | 1E-07 | 1E-07 | 1e-04 |

Table A6. Computational efficiency of non-Darcy AWH flow

Testing WH with Non-darcy Flow (Dec 11, 2025)

The coding for non-Darcy flow multi-phase WH is complete for liquid, gas, and heat with pure water. A couple of bugs were discovered while designing a simulation that would include gridblocks exhibiting liquid, vapor, and two-phase conditions with a single simulation.  The tests for this set of fluid states will use the same 1D geometry and rock parameters described in the AWH tests above, with the exception that the maximum capillary pressure is 0.1 MPa. The boun macro was used to change water flow rates and temperature conditions to produce all phase combinations. A large non-Darcy coefficient (including 1.0E12) was used to ensure the numerical stability of the non-Darcy formulation.

Table A7 summarizes the boundary and initial conditions. The initial conditions and left boundary conditions initiate the simulation in a liquid-only state, resulting in an increasing pressure over time. At 50 days, the water flow rate decreases, and the temperature on the left boundary increases, resulting in a two-phase state.  At 50 days, the water flow rate is increased and the temperature BC is changed from 180 °C to 250 °C, producing conditions that change many gridblocks to a gas-only condition.

| Time (days) | Left BCs (Wflow(kg/s), T(C) | Right BCs (constant Ptot(Mpa), T(C)) | Pg (initial) | Ptot (initial) | T (initial) | Saturation (initial) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0 | (1.e-3,125) | (0.2,90) | NA | 0.2 | 119.5 | 1.0 (liquid) |
| 50 | (1.e-5,180) | (0.2,90) | NA | NA | NA | NA |
| 100 | (1.e-4,250) | (0.2,90) | NA | NA | NA | NA |

Table A7. Boundary and initial conditions for AWH runs for Darcy flow and non-Darcy flow. Ptot is the Total Fluid pressure (Mpa), T is the temperature (°C), and Wflow(kg/s) is the water flow. The initial temperature is below the boiling temperature at the vapor pressure (Ptot).

Tables A13, A14, and A15 show the temperature VS distance at 3 times  for  Darcy flow and non-Darcy flow with ND = 1.d12.

Figure A13. Temperature Vs distance for Darcy (ND00) and non-Darcy flow (ND12)

Figure A14. Saturation vs distance for Darcy (ND00) and non-Darcy flow (ND12)

| Run ID | Newton-Raphson iterations | Solver iterations | Time steps | MBE (H2O) | EBE |  |
| --- | --- | --- | --- | --- | --- | --- |
| ND00 | 308 | 308 | 90 | 1E-06 | 1E-05 |  |
| ND09 | 317 | 317 | 90 | 1E-07 | 1E-05 |  |
| ND11 | 317 | 317 | 90 | 1E-06 | 1E-05 |  |
| ND12 | 654 | 654 | 187 | 1E-06 | 1E-05 |  |

Table A8. Computational efficiency of non-Darcy WH flow

Testing AW2P with Non-darcy Flow (Dec 20, 2025)

The coding for non-Darcy flow multi-phase air-water (AW2P) is complete for liquid water and air. A couple of bugs were discovered while designing a simulation that would include gridblocks exhibiting liquid, vapor, and two-phase conditions with a single simulation.  The test for this set of fluid states will utilize the geometry and 2D geometry, along with rock parameters, as described in the Richards test in the V+V suite, with the exception that a full two-phase simulation will be employed (as opposed to the Richards solution). This problem is characterized by infiltration into a 2D block 100 m wide and 322 m deep. Complex soil and rock permeability as well as van Genuchten relative permeabilities provide an opportunity to test the ND coding with counterflow and highly nonlinear relative permeabilities. Counter current flow wae established with water recharge on the top boundary, and an air injection source on the bottom boundary. Because of the 2-phase nature of the simulations and the high capillary forces, the liquid and gas velocities are relatively low compared with other ND tests.

The details of the geometry, boundary conditions, rock and fluid properties, are described in the V+V description. Figure A14 shows the complexity of the saturation. Even with the large ND coefficient, the saturation shows only a small change at 10000 days between the ND and ND OFF simulations. See the upper curve at 125 and 225m.

Figure A14. Saturation at the midline (x=50m) for ND OFF (ND00) and ND=1.d12(ND12) at 500 and 10000 days with the AW2P ND formulation.

Comparison of the numerical efficiency for non-Darcy and Darcy flow is given in Table A9. All the performance measures are close as expected because of the relatively low flow rates

| Run ID | Newton-Raphson iterations | Solver iterations | Time steps | Phase changes (total) | MBE (H2O) | MB(Air) |
| --- | --- | --- | --- | --- | --- | --- |
| ND00 | 702 | 11365 | 148 | 34 | 1E-05 | 1E-02 |
| ND11 | 697 | 11397 | 152 | 34 | 1E-05 | 1E-02 |
| ND12 | 722 | 12399 | 145 | 36 | 1E-05 | 1E-02 |

Table A9. Computational efficiency of non-Darcy AW2P flow

Remarks

Conclusions